Your ref: Please Reply To: Senior Information Governance Officer T 01924 306112 E: freedomofinformation@wakegield.gov.uk Date: 15/05/2026

Transformation Directorate

Service Director: Emma Marshall

Wakefield One

Burton St

Wakefield

WF1 3EB

Typetalk calls welcome

# Contact: Senior Information Governance Officer

# T 01924 306112

# E: freedomofinformation@wakegield.gov.uk:

# Typetalk calls welcome

From a textphone dial: 18001 0345 8 503 503
From a telephone dial: 18002 0345 8 503 503

Dear Requester,

Environmental Information Regulations 2004 - Request for Information

REF: EIR 23107

I am writing in respect of your enquiry for information held by the Authority, which has been processed under the provisions of the above Regulations. Please note that, as previously advised, the EIR legislation is relevant, as your request relates to issues which affect the environment.

Please find the response to your request below.

Can I ask where the waste then goes from Biffa waste management facility at South Kirkby?

General household waste is tipped into the Biffa Residual Waste Treatment Facility in South Kirkby.

Within this facility the waste is deposited into the reception hall and then loaded into a shredder before being transferred into our pre-sort area of the facility via conveyor. Within the pre-sort, waste is separated into the following streams:

·         Plastic bottles – these are then baled and sent to a recycling re-processor.

·         Steel / aluminium cans – loaded into containers and sent to a recycling re-processor.

·         RDF (Refuse Derived Fuel) – this is the combustible element of the waste such as plastic food trays, plastic film, dirty paper / card. This is bulked up and sent to Ferrybridge for combustion to produce power.

·         Organic – this material is fed further through the facility to a pre-treatment area.

When the organic material enters the pre-treatment area it passes through various mechanical processes to extract any glass, stone, sand, grit, small metal / plastic items before being pumped into the Anaerobic Digestion tanks. Within these tanks the waste breaks down under the anaerobic conditions producing gas which is then fed into two engines to produce electricity which is then used across the site with any excess being exported to the grid.

Once the material within the tanks has stopped yielding gas it is then dried out to produce a digestate material that is sent out for spreading on fields (i.e. farms).

Building waste from households should be taken to the Household Waste Recycling Centre and dependent on the quality of the material it will either be sent for recycling or to an inert landfill site.

Can I ask when you say there are no other waste collection companies registered with Wakefield Council, where do the small waste companies take their waste?

There are some small local waste companies who are registered to tip waste at Biffa’s waste transfer station on Denby Dale Road in Wakefield.

I trust that this information is satisfactory for your purposes.

If you are not satisfied with the way the Authority has handled your request, you may ask for an internal review. Please contact:

Senior Information Governance Officer
Corporate Information Governance Team
County Hall
Wakefield
West Yorkshire
WF1 2QW

Or email: freedomofinformation@wakefield.gov.uk

Yours Sincerely,

Information Governance Officer

Information Governance Team,

Wakefield Council

E-mail: freedomofInformation@wakefield.gov.uk

Tel: 01924 306112